CCNA LAB — DAY 1

Zero Trust Network — Build & Configuration Guide

Date: April 27, 2026  |  Packet Tracer Lab

# 1. Lab Overview

This lab implements a Zero Trust Network (ZTN) architecture in Cisco Packet Tracer. The design follows the core Zero Trust principle of never trust, always verify — every device, user, and traffic flow is treated as potentially hostile regardless of its location in the network.

The network is divided into three security zones enforced by three Zero Trust Points:

- ZT Point 1 — Edge-FW: deny-all inbound ACL from the internet

- ZT Point 2 — DMZ: isolated VLAN 50 for public-facing servers

- ZT Point 3 — Core-Router: inter-VLAN microsegmentation ACLs

# 2. Device Inventory

| Device | Model | Role | Zone |
| --- | --- | --- | --- |
| ISP-Router | ISR 4321 | Simulates internet / ISP handoff | Internet |
| Edge-FW | ISR 4321 + NIM-2T | Edge firewall, ZT Point 1 | Edge |
| DMZ-SW | 2960-24TT | DMZ switch, VLAN 50 | DMZ |
| Web-Server | Server-PT | HTTP/HTTPS web server | DMZ |
| DNS-Server | Server-PT | DNS resolver | DMZ |
| Core-Router | ISR 4321 + NIM-2T | Inter-VLAN routing, ZT Point 3 | Internal |
| Internal-SW | 2960-24TT | Internal switch, VLANs 10/20/30/100 | Internal |
| Admin-PC | PC-PT | IT Admin workstation — VLAN 10 | Internal |
| Staff-PC | PC-PT | Staff workstation — VLAN 20 | Internal |
| IoT-PC | PC-PT | IoT/Guest device — VLAN 30 | Internal |
| AAA-Server | Server-PT | RADIUS authentication server | Internal |
| Syslog-Server | Server-PT | Log collection server | Internal |

# 3. IP Addressing Plan

| Device | Interface | IP Address | Subnet |
| --- | --- | --- | --- |
| ISP-Router | Gi0/0/0 | 203.0.113.1 | /30 |
| Edge-FW | Gi0/0/0 (WAN) | 203.0.113.2 | /30 |
| Edge-FW | Gi0/0/1.50 (DMZ) | 192.168.50.1 | /24 |
| Edge-FW | Se0/1/0 (to Core) | 10.0.0.1 | /30 |
| Core-Router | Se0/1/0 | 10.0.0.2 | /30 |
| Core-Router | Gi0/0/0.10 (VLAN 10) | 192.168.10.1 | /24 |
| Core-Router | Gi0/0/0.20 (VLAN 20) | 192.168.20.1 | /24 |
| Core-Router | Gi0/0/0.30 (VLAN 30) | 192.168.30.1 | /24 |
| Core-Router | Gi0/0/0.100 (Servers) | 192.168.100.1 | /24 |
| Web-Server | Fa0 | 192.168.50.10 | /24 |
| DNS-Server | Fa0 | 192.168.50.20 | /24 |
| Admin-PC | Fa0 | 192.168.10.10 | /24 |
| Staff-PC | Fa0 | 192.168.20.10 | /24 |
| IoT-PC | Fa0 | 192.168.30.10 | /24 |
| AAA-Server | Fa0 | 192.168.100.10 | /24 |
| Syslog-Server | Fa0 | 192.168.100.20 | /24 |

# 4. Cable Connections

| From | Port | To | Port | Cable Type |
| --- | --- | --- | --- | --- |
| ISP-Router | Gi0/0/0 | Edge-FW | Gi0/0/0 | Copper Straight-Through |
| Edge-FW | Gi0/0/1 | DMZ-SW | Fa0/1 | Copper Straight-Through |
| Edge-FW | Se0/1/0 (DCE) | Core-Router | Se0/1/0 (DTE) | Serial DCE |
| DMZ-SW | Fa0/2 | Web-Server | Fa0 | Copper Straight-Through |
| DMZ-SW | Fa0/3 | DNS-Server | Fa0 | Copper Straight-Through |
| Core-Router | Gi0/0/0 | Internal-SW | Gi0/1 | Copper Straight-Through |
| Internal-SW | Fa0/2 | Admin-PC | Fa0 | Copper Straight-Through |
| Internal-SW | Fa0/3 | Staff-PC | Fa0 | Copper Straight-Through |
| Internal-SW | Fa0/4 | IoT-PC | Fa0 | Copper Straight-Through |
| Internal-SW | Fa0/5 | AAA-Server | Fa0 | Copper Straight-Through |
| Internal-SW | Fa0/6 | Syslog-Server | Fa0 | Copper Straight-Through |

# 5. VLAN Design

| VLAN ID | Name | Subnet | Devices | Access Level |
| --- | --- | --- | --- | --- |
| 10 | IT-ADMIN | 192.168.10.0/24 | Admin-PC | Full access |
| 20 | STAFF | 192.168.20.0/24 | Staff-PC | Limited — no VLAN 10 or 30 |
| 30 | IOT-GUEST | 192.168.30.0/24 | IoT-PC | Internet only — no internal VLANs |
| 50 | DMZ | 192.168.50.0/24 | Web-Server, DNS-Server | Public-facing only |
| 100 | SERVERS | 192.168.100.0/24 | AAA-Server, Syslog-Server | Internal servers |

# 6. Router Configuration

## 6.1 ISP-Router

- hostname ISP-Router

- interface GigabitEthernet0/0/0

- ip address 203.0.113.1 255.255.255.252

- no shutdown

- ip route 0.0.0.0 0.0.0.0 203.0.113.2

- ip domain-name zerotrust.lab

- crypto key generate rsa  [1024 bits]

- ip ssh version 2

- username admin secret Cisco123!

- line vty 0 4

- login local

- transport input ssh

- exec-timeout 5 0

## 6.2 Edge-FW

- hostname Edge-FW

- interface GigabitEthernet0/0/0

- description WAN-TO-ISP

- ip address 203.0.113.2 255.255.255.252

- no shutdown

- ip access-group ZT-INBOUND in

- interface GigabitEthernet0/0/1

- description TO-DMZ

- no ip address

- no shutdown

- interface GigabitEthernet0/0/1.50

- description DMZ-VLAN50

- encapsulation dot1Q 50

- ip address 192.168.50.1 255.255.255.0

- interface Serial0/1/0

- description TO-CORE-ROUTER

- ip address 10.0.0.1 255.255.255.252

- clock rate 64000

- no shutdown

- ip route 0.0.0.0 0.0.0.0 203.0.113.1

- ip route 192.168.0.0 255.255.0.0 10.0.0.2

## 6.3 Core-Router

- hostname Core-Router

- interface Serial0/1/0

- description TO-EDGE-FW

- ip address 10.0.0.2 255.255.255.252

- no shutdown

- interface GigabitEthernet0/0/0

- description TO-INTERNAL-SW

- no ip address

- no shutdown

- interface GigabitEthernet0/0/0.10

- encapsulation dot1Q 10

- ip address 192.168.10.1 255.255.255.0

- ip access-group ZT-VLAN20-STAFF in  [applied on .20]

- interface GigabitEthernet0/0/0.20

- encapsulation dot1Q 20

- ip address 192.168.20.1 255.255.255.0

- ip access-group ZT-VLAN20-STAFF in

- interface GigabitEthernet0/0/0.30

- encapsulation dot1Q 30

- ip address 192.168.30.1 255.255.255.0

- ip access-group ZT-VLAN30-IOT in

- interface GigabitEthernet0/0/0.100

- encapsulation dot1Q 100

- ip address 192.168.100.1 255.255.255.0

- ip route 192.168.50.0 255.255.255.0 10.0.0.1

- ip route 0.0.0.0 0.0.0.0 10.0.0.1

# 7. Zero Trust ACL Configuration

## 7.1 Edge-FW — ZT-INBOUND ACL

Applied inbound on Gi0/0/0 (WAN interface). Blocks all internet traffic except permitted DMZ services.

- ip access-list extended ZT-INBOUND

- permit tcp any host 192.168.50.10 eq www

- permit tcp any host 192.168.50.10 eq 443

- permit udp any host 192.168.50.20 eq domain

- deny ip any any

## 7.2 Core-Router — ZT-VLAN20-STAFF ACL

Applied inbound on Gi0/0/0.20. Limits Staff VLAN access — permits AAA, Syslog, DMZ web. Blocks lateral movement to IT Admin and IoT.

- ip access-list extended ZT-VLAN20-STAFF

- permit tcp 192.168.20.0 0.0.0.255 host 192.168.100.10 eq 49

- permit udp 192.168.20.0 0.0.0.255 host 192.168.100.20 eq 514

- permit tcp 192.168.20.0 0.0.0.255 host 192.168.50.10 eq 80

- permit tcp 192.168.20.0 0.0.0.255 host 192.168.50.10 eq 443

- deny ip 192.168.20.0 0.0.0.255 192.168.10.0 0.0.0.255

- deny ip 192.168.20.0 0.0.0.255 192.168.30.0 0.0.0.255

- permit ip any any

## 7.3 Core-Router — ZT-VLAN30-IOT ACL

Applied inbound on Gi0/0/0.30. IoT devices are internet-only — all internal VLANs blocked.

- ip access-list extended ZT-VLAN30-IOT

- deny ip 192.168.30.0 0.0.0.255 192.168.10.0 0.0.0.255

- deny ip 192.168.30.0 0.0.0.255 192.168.20.0 0.0.0.255

- deny ip 192.168.30.0 0.0.0.255 192.168.100.0 0.0.0.255

- deny ip 192.168.30.0 0.0.0.255 192.168.50.0 0.0.0.255

- permit ip any any

# 8. SSH Hardening

Configured on all three routers. Telnet disabled on all VTY lines. SSH version 2 enforced with RSA 1024-bit keys.

- ip domain-name zerotrust.lab

- crypto key generate rsa  [select 1024 when prompted]

- ip ssh version 2

- username admin secret Cisco123!

- line vty 0 4

- login local

- transport input ssh

- exec-timeout 5 0

- line console 0

- exec-timeout 10 0

# 9. Verification Tests

| Test | Command | From | Expected | Result |
| --- | --- | --- | --- | --- |
| VLAN 10 gateway | ping 192.168.10.1 | Admin-PC | Success | PASS |
| VLAN 20 gateway | ping 192.168.20.1 | Staff-PC | Success | PASS |
| VLAN 30 gateway | ping 192.168.30.1 | IoT-PC | Success | PASS |
| Inter-VLAN routing | ping 192.168.20.10 | Admin-PC | Success | PASS |
| Servers reachable | ping 192.168.100.10 | Admin-PC | Success | PASS |
| DMZ reachable | ping 192.168.50.10 | Admin-PC | Success | PASS |
| Internet edge | ping 203.0.113.1 | Admin-PC | Success | PASS |
| ZT inbound block | telnet 192.168.100.10 80 | ISP-Router | Fail | PASS |
| ZT DMZ permit | telnet 192.168.50.10 80 | ISP-Router | Connects | PASS |
| Staff blocked from IT | ping 192.168.10.10 | Staff-PC | ACL deny (4 matches) | PASS |
| IoT blocked from Staff | ping 192.168.20.10 | IoT-PC | ACL deny (4 matches) | PASS |
| SSH login works | ssh -l admin 192.168.10.1 | Admin-PC | Login prompt | PASS |
| Telnet blocked | telnet 192.168.10.1 | Admin-PC | Connection closed | PASS |

# 10. CCNA Topics Covered

- VLANs — creation, naming, port assignment on 2960 switches

- Trunking — 802.1q encapsulation, trunk allowed VLANs

- Inter-VLAN routing — Router-on-a-Stick with subinterfaces and dot1Q

- Static routing — default routes and specific network routes

- Serial interfaces — DCE/DTE roles, clock rate configuration

- Named extended ACLs — source/destination, protocol, port filtering

- ACL placement — inbound vs outbound, interface application

- SSH hardening — RSA key generation, VTY transport input ssh

- Network segmentation — Zero Trust microsegmentation design

- Troubleshooting — show commands, ACL match counters, interface status